Chapter 4 + 5

1- What is the Layered Approach to Computer Architecture?

1- Application level. (Calculator)

2- High-level language level. (

3- Operating system.

4- Assembly code level.

5- Machine code level.

6- Microcode level.

7- Logic level.

8- Technology level

2- What are the basic logical gates?

1- Buffer: used for drive several gates or devices requiring higher drive requirements.

2- Inverter or NOT

3- AND.

4- NAND.

5- OR.

6- NOR.

7- XOR.

3- Rules of Boolean algebra:

1- Identity Proposition

A + 0 = A

A + 1 = 1

A·1 = A

A·0 = 0

2- Inverse Proposition

A + A = 1

A · A = 0

3- Commutative Proposition

A · B = B · A

A + B = B + A

4- Distributive Proposition

A·(B + C) = A·B + A·C

A + B·C = (A + B) · (A + C)

5- De Morgan's Theorem

A + B = A • B

A • B = A + B

4- Write truth table of 1-Bit Half Adder

| C | S | B | A |
| --- | --- | --- | --- |
| 0 | 0 | 0 | 0 |
| 0 | 1 | 1 | 0 |
| 0 | 1 | 0 | 1 |
| 1 | 0 | 1 | 1 |

A

B

S

C

1- Combinatorial logic: A combinatorial circuit consists of n binary inputs and m binary outputs

2- 7-Segment Decimal Decoder: Turning specific segments on allows the display of all 10 decimal digits.

3- Karnaugh Maps: A Karnaugh Map is a diagram on which we can plot the sum of products form of a Boolean expression. Allows the elimination and reduction of terms to be made visually. A K-map for n variables contains 2n squares.

4- Multiplexer: Connects one of multiple inputs to the single output and Multiplexers are used to control signal and data routing.

5- Demultiplexer: Performs the opposite function to a multiplexer. Connect one input and multiple outputs.

6- Decoder: Similar to demultiplexer except there is no input line.

7- PLA: it stands of Programmable Logic Arrays. Useful for implementing combinatorial functions expressed as sums of products.