Chapter 1

1- What are the definitions of?

1- Communication: Imparting or conveying or exchange information

2- protocol: A set of rules that govern communication

3- Data Communication: Transimation media and protocol to exchange data between digital devices.

F=1/P	                    4- Frequency: The number of periods per second

5- Amplitude: The value between which the signals oscillation

6- Phase: The Position of the waveform relative to time Zero

7- Wavelength: The speed of light divided by Frequency       wave= speed/F

8- Bandwidth: The difference between highest and lowest frequency in a signal

9- Data Rate: The Number of bits transmitted per second

10- Microwave: Require no obstacles between transmitter and receiver and the data rate is up to 300MBPS

2- What are the types of Signals?

1- Digital Signals: Represent by an alternative sequence of high or low value.

2- Analog Signals: Varies continuously in a range between values.

3- What are the Types of Media?

1- Twisted Pair: Two copper wire are twisted around each other

2- Coaxial Cable: Transmits information in either base band or broadband

3- Optical Fiber: use alight to transmit information.

4- What are the consists of Coaxial Cable?

1- Copper of Aluminum conductress

2- Insulator

3- Wire mesh

4- Outer cover

5- Comparing between Types of Media?

| Optical Fiber | Coaxial Cable | Twisted Pair | Face Comparing |
| --- | --- | --- | --- |
| 1-carry much information 2- use in Telephone Company 3- data security Let information to be leaked 4- electromagnetic interface | 1- greeter bandwidth 2- broadband system 3- greeter channel capacity 4- high installation cost | 1- large frequency 2- reduce crosstalk | Advantages |
| 1- price 2- fragility 3- problem underwater cable 4- require special skills such as additional Training | 1- more expensive 2- very bulky 3- high attenuation 4- not support for some network | 1- attention problem 2- relative low bandwidth | Disadvantages |
|  | 500 MHZ | 250 KHZ | Bandwidth |
| Up to 28 GBPS | Up to 800 MBPS | 100 MBPS | Data Rate |